Edge AI CCTV - Hardware Requirements & BOM (AUD)

Architecture: 100% On-Premise Edge Surveillance (Zero Cloud Costs)
Primary Compute: Intel N100 Mini PC (8 GB RAM + 256 GB SSD) using built-in Intel UHD iGPU
Currency: Australian Dollars (AUD) — Verified from live Australian retailers (Amazon AU, Umart, Scorptec, Core Electronics, Jaycar).

1. Necessary Items (Core Edge System)

| Item | Use / Need | Cost (AUD) |
| --- | --- | --- |
| Intel N100 Mini PC (GMKtec G3 / Beelink S12 Pro) | Host Edge Server: Quad-Core CPU (up to 3.4GHz, 6W TDP) with Intel UHD QuickSync hardware video decoding (VA-API H.264/H.265). Runs FastAPI backend, SQLite DB, DVR engine, and WebRTC streaming. | 99.00 |
| RAM: 8 GB DDR4 / DDR5 (3200MHz / 4800MHz SODIMM) | System Memory: Buffers incoming camera RTSP streams, maintains in-memory JPEG ring buffers, and caches SQLite event logs. Uses ~2.2GB–3.0GB active RAM, leaving ~5GB free. | Included (with PC) |
| Storage: 256 GB – 512 GB SSD (Crucial P3 Plus / Kingston NV2) | System Drive & 24/7 DVR: Houses OS, AI models, SQLite database, incident video clip archives, and rolling continuous DVR recording segments. | Included (with PC) |
| Integrated GPU (Intel UHD 24 EUs) (OpenVINO FP16 Engine) | AI Inference (1–4 Cameras): Built-in iGPU runs YOLO11 multi-class object detection and pose estimation directly in OpenVINO. Zero additional cost (Hailo module avoided). | bash.00 (Included) |
| 5-Port Gigabit PoE+ Switch (TP-Link TL-SG1005P, 65W PoE) | Camera & Network Power: Delivers data transmission and direct power to PoE IP cameras over single Cat6 cables without separate power adapters. | 5.00 |
| Wi-Fi 6 Router / Access Point (TP-Link Archer AX12 / AX23) | Local Wireless Gateway: Serves encrypted LAN streams, Web HUD dashboard, and mDNS auto-discovery for mobile apps and ESP32 wireless cameras. | 9.00 |
| PoE IP Camera (4MP / 5MP) (Reolink RLC-520A / Dahua 4MP) | Primary Video Feed: High-resolution outdoor RTSP video stream with infrared night vision and weatherproof (IP67) housing. | 9.00 |
| Cabling & Power Infrastructure (Cat6 Patch Leads 15m + PDU) | Physical Connectivity: Connects IP cameras to PoE switch and edge host to the router. | 8.00 |
| Total (Base Necessary System) | Complete Standalone Edge AI Surveillance Hub | ~60.00 AUD |

2. Optional Upgrades & Components

| Item | Use / Need | Cost (AUD) |
| --- | --- | --- |
| RAM Upgrade: 16 GB DDR4 / DDR5 (16GB SODIMM or PC Variant) | Multi-Camera Expansion & Heavy Docker Load: Recommended when scaling beyond 4 cameras, buffering multiple concurrent 4K high-bitrate streams, or running heavy parallel microservices. | 5.00 |
| Hailo-8L M.2 AI Module (13 TOPS) (M.2 Key M / B+M 2242/2280) | Additional Cameras (4 to 8 Feeds): Offloads AI inference from host iGPU to dedicated NPU hardware; enables sub-10ms YOLO11 detection across multiple additional camera channels. | 25.00 |
| Hailo-8 M.2 AI Module (26 TOPS) (M.2 Key M 2280) | Commercial Multi-Camera Expansion (8 to 16 Feeds): Heavy-duty NPU upgrade running up to 16 concurrent camera feeds with YOLO11 detection and 17-keypoint pose tracking simultaneously. | 59.00 |
| ESP32-S3 Camera Sentry Node (Freenove S3 / Seeed XIAO Sense) | Satellite Camera & IoT Sensor Hub: Wireless Wi-Fi video stream (800x600 SVGA) with breakout GPIO pins for physical sentry sensors. | 2.00 |
| PIR Motion Sensor (HC-SR501 / AM312) | Physical Intrusion Detection: Passive infrared human body heat detector; triggers instant edge alert webhooks and 15s DVR incident clips. | .50 |
| Ultrasonic Distance Sensor (HC-SR04P) (3.3V Native Logic) | Proximity & Perimeter Breach: Measures physical distance (2cm – 400cm); detects vehicles in driveway or people approaching doorways. | .50 |
| Magnetic Door Reed Switches (MC-38, 2x) | Door / Gate Contact Monitoring: Simple 2-wire magnetic switches connected to ESP32 pullup pins (monitors front door, back door, or lock state). | .00 |
| Mini UPS Battery Backup (650VA / 360W) (CyberPower / APC Back-UPS) | Power Outage Protection: Provides 45–60 minutes of clean backup power to Edge PC, switch, and cameras during grid blackouts or wire tampering. | 9.00 |
| 2 TB Surveillance Storage Expansion (WD Purple 2TB / Crucial P3 2TB) | Long-Term DVR Retention: Expands continuous 24/7 video storage from 3–5 days to 30+ days across multiple camera channels. | 15.00 |
| Active 5V Buzzer / Strobe Relay | Physical Alarm Deterrent: Connects to ESP32 pin to trigger an audible local siren or strobe light upon perimeter breach. | .50 |
| Weatherproof Sentry Enclosure (ABS Junction Box IP65) | Outdoor Sensor Housing: Houses ESP32-S3 module, PIR sensor, and ultrasonic transducer for outdoor weather protection. | 4.00 |